SIH26126

Vision-Based Autonomous Navigation for a GPS-Denied Outdoor UGV

FULL IMPLEMENTATION ROADMAP

Software-first build -> hardware bench validation -> hardware-in-the-loop -> integrated autonomy -> measured SIH validation

Purpose A process-to-process master build plan using the SIH PS 26126 and the latest locked hardware/software/pipeline documents. It separates what is frozen from what must be measured and from what is only an implementation organization choice.

Primary source set: SIH PS 26126; FINAL_LOCKED_UGV_BUILD_DOCUMENT(2); software_validation_rover_final_design; Wire-to-Wire_Pipeline_A-B-C_FINAL(4); FINAL_UGV_BOM_PRICES_ALL_CONNECTIONS_ARCHITECTURE(1).

1. Source of Truth and Non-Hallucination Rules

| ID | Source | Authority |
| --- | --- | --- |
| S1 | SIH PS 26126 | Problem requirements and success criterion. |
| S2 | FINAL_LOCKED_UGV_BUILD_DOCUMENT(2).pdf | Final hardware, GPIO, power, safety, mechanics, purchase-time checks. |
| S3 | software_validation_rover_final_design.docx | Firmware sequence, physical layout, bench test order, compatibility conditions. |
| S4 | Wire-to-Wire_Pipeline_A-B-C_FINAL(4).docx | Stages 0-33, algorithms, formulas, interfaces, validation, failure modes. |
| S5 | FINAL_UGV_BOM_PRICES_ALL_CONNECTIONS_ARCHITECTURE(1).pdf | Detailed BOM/procurement and connection cross-check. |

Authority rule Use the latest locked build document for physical procurement whenever an older document differs. The latest lock explicitly supersedes earlier drafts. Do not revive older GPIO maps or older BOM variants.

| Tag | Meaning |
| --- | --- |
| LOCKED | Directly stated as final in the source. |
| MEASURE | Must be physically/experimentally established; do not guess. |
| IMPLEMENTATION CHOICE | Not frozen by the source; isolate behind an interface and document the selection. |
| NOT PROVEN | Specified by design but not yet executed against actual hardware/data. |
| OPTIONAL | Can be added later without changing the frozen core path. |

The wire-to-wire source explicitly states that its complete A+B+C architecture is a map of how the system is meant to work, not evidence that the system already runs; evidence comes only from execution and logging. This roadmap preserves that distinction.

2. SIH PS -> Implementation Acceptance Map

| PS requirement | Our implementation | Evidence |
| --- | --- | --- |
| Path detection | MiDaS-Small -> scale anchor -> 3D projection -> RANSAC ground plane -> signed-distance hazard labels -> traversability grid | Frame overlays, grid packets, confidence, RUGD/RELLIS validation. |
| Visual localization | Camera/IMU sync -> ORB -> essential geometry -> ORB-SLAM3 -> IMU preintegration -> mapping -> loop closure -> pose confidence | Pose log, tracking state, ATE/RPE, revisit run. |
| Collision avoidance | Timestamp gate -> world frame -> rolling costmap -> log-odds + footprint inflation -> reactive gap -> bicycle constraint -> cubic curve -> recovery | Obstacle/costmap tests, curve test, recovery test, watchdog tests. |
| Wheel/motor commands | Pure Pursuit -> rate limit/speed reduction -> Wi-Fi -> ESP32 -> TB6612 + MG90S | Bench command test and low-speed closed-loop run. |
| Point A -> Point B | Recomputed closed-loop navigation with measured course | Repeated success rate, path length and SPL inputs. |

System classification Hybrid autonomy: lightweight AI perception + classical geometric reasoning + visual-inertial SLAM + classical local planning/control + deterministic onboard safety.

3. Frozen Architecture and Exact Build Order

End-to-end runtime order:

CAMERA+IMU -> A: preprocess/calibrate/depth/geometry/grid -> A packet CAMERA+IMU -> B: sync/ORB/SLAM/VIO/map/loop/pose -> B packet A packet + B packet -> C: timestamp gate -> world transform -> rolling costmap -> occupancy/inflation -> reactive gap -> bicycle constraint -> cubic curve -> Pure Pursuit -> rate limiting -> Wi-Fi command -> ESP32 -> motors/servo IN PARALLEL: ultrasonic + Wi-Fi timeout + E-stop -> independent watchdog -> forced stop

Build order:

| Phase | Work | Gate |
| --- | --- | --- |
| 0 | Environment, repo, config, logging | Reproducible laptop build. |
| 1 | Camera + calibration | K/D and replay pass. |
| 2 | Member A | A packet replay pass. |
| 3 | ORB-SLAM3 + sync | Benchmark + project camera/IMU pass. |
| 4 | Member B | B packet replay pass. |
| 5 | Member C | Math + synthetic packet tests pass. |
| 6 | ESP32-CAM/WROOM firmware | All bench firmware tests pass. |
| 7 | Power/sensors/actuators | Physical bring-up pass. |
| 8 | Hardware-in-the-loop | Low-speed command + watchdog pass. |
| 9 | Full A+B+C + calibration | Closed-loop latency measured. |
| 10 | Outdoor + datasets + metrics | Evidence package complete. |
| 11 | SIH demo | Repeatable Point A -> B run. |

4. Laptop Environment, Acquisition, and Repository

| Layer | Use | Implementation |
| --- | --- | --- |
| Python | Member A orchestration + Member C logic/logging | Primary high-level application layer. |
| C++ | ORB-SLAM3 core + thin adapter | Build official ORB-SLAM3 source; expose pose packets. |
| OpenCV | Capture, calibration, undistortion, Sobel, image utilities | Shared camera-geometry library. |
| MiDaS-Small | Monocular depth AI | Use selected pretrained checkpoint; freeze exact model identity. |
| ORB-SLAM3 | Monocular-inertial SLAM | Use DBoW2 loop closing + bundle adjustment. |
| ESP32 Arduino/C++ | Low-level firmware | GPIO/I2C/LEDC/Wi-Fi/watchdog. |
| Logs | Per-frame + per-run evidence | CSV/JSON-style records; keep raw artifacts. |

Acquisition rules:

Use the official MiDaS repository/release for the chosen pretrained checkpoint and record the exact checkpoint identity used.

Use the official ORB-SLAM3 source/build path; first prove the build and benchmark examples before integrating the project camera/IMU.

Use the official Espressif Arduino-ESP32 toolchain/documentation for ESP32 installation, Wi-Fi and LEDC PWM.

Pin versions/commits after the first successful build so later dependency updates cannot silently change performance.

ugv_autonomy/ config/ member_a/ member_b/ member_c/ rover/esp32_wroom/ rover/esp32_cam/ tests/unit/ tests/integration/ tests/hardware_in_loop/ datasets/ calibration/ logs/ reports/

Transport note The source locks Wi-Fi but does not lock a specific TCP/UDP/HTTP transport. Keep transport behind command_sender/receiver interfaces; choose and freeze the transport after measuring latency and packet loss.

5. Member A - Perception Build Sheet (Stages 0-11)

| Stage | Purpose | Input -> Process | Output | Math / method | Build + validation |
| --- | --- | --- | --- | --- | --- |
| S0 | Physical inputs | RGB frame + timestamp; IMU ax,ay,az,gx,gy,gz + timestamp | Timestamped input record; camera/IMU abstractions | Camera + MPU6050 | Implement replayable input layer before hardware. |
| S1 | Preprocess | Raw RGB + D | Resize to calibrated resolution; cv2.undistort; preserve timestamp | OpenCV capture/undistortion | Resolution must exactly match calibration. |
| S2 | Camera calibration | 15-25 checkerboard images; 25 mm squares | Find corners; cv2.calibrateCamera; save K,D,resolution,FPS | OpenCV checkerboard | K=[[fx,0,cx],[0,fy,cy],[0,0,1]]; D=[k1,k2,p1,p2,k3] |
| S3 | MiDaS-Small | Undistorted RGB | Normalize/resize -> pretrained MiDaS-Small -> depth map | Monocular depth NN | Raw output is relative, not metres; benchmark real latency. |
| S4 | Confidence | Frame + later RANSAC inlier ratio | Sobel gradient variance + exposure + inlier ratio -> consecutive-frame LOW_CONFIDENCE | Heuristic failure detector | texture_score=variance(¦gradient(I)¦) |
| S5 | Scale anchor | MiDaS depth + camera height/tilt | Estimate expected ground depth; median ratio; scale full map | Geometric anchoring | s=median(depth_expected/depth_midas_raw); d=s*d_raw |
| S6 | Pinhole projection | u,v,Z,K | Sample points -> compute camera-frame XYZ | Pinhole geometry | X=(u-cx)Z/fx; Y=(v-cy)Z/fy; Z=depth(u,v) |
| S7 | Camera->vehicle | 3D camera point + fixed R,t | Apply rigid transform from measured mount geometry | Rigid transform | P_vehicle=R*P_camera+t |
| S8 | Ground plane | Vehicle-frame 3D points | Lower-region candidates -> 3-point RANSAC 50-100 iterations -> inlier LS refinement | RANSAC plane fitting | aX+bY+cZ+d=0 |
| S9 | Hazard class | 3D point + plane | Signed distance -> T_up/T_down -> SAFE/OBSTACLE/DROP_OFF -> remove isolated noise | Signed plane distance | dist=(aX+bY+cZ+d)/sqrt(a^2+b^2+c^2) |
| S10 | Traversability grid | Hazard-labeled 3D points | Map to 2D cells; conservative label wins; UNKNOWN for unsampled cells; attach metadata | Grid aggregation | Resolution chosen by smallest needed hazard + compute budget |
| S11 | A->C packet | Timestamp; grid; resolution/scale; confidence; LOW_CONFIDENCE | Serialize to JSON-style packet; log and transmit | Timestamped interface | Keep transport-independent |

Member A acceptance: calibration saved; MiDaS runs; scale anchor checked against tape measurement; RANSAC plane visualized; signed-distance thresholds measured; A packet replay works.

6. Member B - Localization Build Sheet (Stages 12-20)

| Stage | Purpose | Input -> Process | Output | Math / method | Build + validation |
| --- | --- | --- | --- | --- | --- |
| S12 | Sync camera+IMU | Undistorted frames + IMU stream | Collect IMU interval; interpolate to frame timestamp if needed; apply extrinsics | Timestamp sync + extrinsics | T_imu_camera=inv(T_chassis_imu)*T_chassis_camera |
| S13 | ORB features | Synchronized/undistorted frame | FAST keypoints -> oriented BRIEF -> Hamming matches -> reject weak/ambiguous | ORB | Log feature/match counts |
| S14 | Essential geometry | Matched points + K | Estimate F with RANSAC -> E -> SVD/decompose R,t direction | Epipolar geometry | p2^T E p1=0; E=K^T F K; E=[t]x R |
| S15 | ORB-SLAM3 tracking | Matches + local map + IMU constraint | Optimize current pose against map + IMU; emit valid 6-DoF pose | ORB-SLAM3 Tracking | T_wc/T_cw; x,y,z,roll,pitch,yaw |
| S16 | IMU preintegration | IMU samples between keyframes | Accumulate delta_R, delta_v, delta_p; jointly optimize with visual constraints | Tightly coupled VIO | Conceptual terms delta_R,delta_v,delta_p |
| S17 | Local mapping + BA | Keyframes + map points | Triangulate; local nonlinear LS; prune redundant/unreliable map items | Keyframe mapping + bundle adjustment | min sum ¦¦project(point,pose)-observed_pixel¦¦^2 |
| S18 | Loop closure | Keyframe descriptors | DBoW2 retrieval -> geometric verification -> pose-graph correction | DBoW2 + pose graph | Design a revisit segment to prove it. |
| S19 | Pose confidence | Features + reprojection error + IMU consistency + tracking state | Combine evidence; invalidate pose on tracking loss | Composite heuristic | Not a calibrated probability. |
| S20 | B->C packet | Timestamp; pose; confidence; valid; optional trajectory; loop flag | Serialize, transmit, log | Timestamped interface | Never pass stale/invalid pose. |

Member B acceptance: ORB-SLAM3 builds; benchmark run works; camera/IMU synchronization and extrinsics are measured; live project tracking is demonstrated; pose confidence and loop closure are logged.

7. Member C - Planning, Control, Safety, Timing (Stages 21-33)

| Stage | Purpose | Input -> Process | Output | Math / method | Build + validation |
| --- | --- | --- | --- | --- | --- |
| S21 | Timestamp/confidence gate | A grid packet + B pose packet | Nearest timestamp within tolerance; reject stale/low-confidence pair | Nearest timestamp + confidence gate | ¦tA-tB¦<=delta_t_sync |
| S22 | World transform | Matched grid + pose | Rotate/translate every local cell into world frame | 2D rigid transform | xw=xt+xl cos(th)-yl sin(th); yw=yt+xl sin(th)+yl cos(th) |
| S23 | Rolling local window | World cells + vehicle position | Keep cells only within R_local | Fixed-radius rolling window | R_local >= detection_range + reaction_distance + margin |
| S24 | Occupancy + inflation | Bounded cells + observations + measured width | Log-odds update; classify hazard; inflate by half width + margin | Log-odds occupancy + footprint inflation | l_t=l_(t-1)+log(p/(1-p))-l_0; r_inflate=width/2+margin |
| S25 | Reactive gap decision | Inflated costmap | Find corridor wide enough for footprint; feasible -> curve; none -> recovery | VFH/DWA-style local planning | Recompute every frame from live costmap |
| S26 | Bicycle constraint | Measured wheelbase L + max steering delta_max | Compute physical minimum radius | Single-track kinematics | R_min=L/tan(delta_max) |
| S27 | Curve generation | Gap + costmap + R_min + pose | Four endpoint position/heading constraints -> solve cubic -> curvature check | Cubic polynomial | y(x)=a x^3+b x^2+c x+d |
| S28 | Reverse-and-retry | No feasible forward curve + rear clearance | Check rear costmap + rear sensor; reverse short fixed distance; recompute; one retry only | Straight-line recovery | No indefinite reverse |
| S29 | Pure Pursuit | Feasible curve + pose + L | Select lookahead point -> alpha -> steering command | Pure Pursuit | delta=atan(2 L sin(alpha)/l_d) |
| S30 | Rate limit + slow | Desired steering + prior applied + dt | Clip delta change; reduce speed in avoidance | Per-frame rate clipping | delta_app=delta_prev+clip(delta_des-delta_prev,-max_rate*dt,+max_rate*dt) |
| S31 | Wi-Fi command | Rate-limited steering + speed | Package -> send -> ESP32 validates freshness -> apply | Wi-Fi command interface | Transport not frozen by source; isolate it. |
| S32 | Independent watchdog | Command age + ultrasonic + E-stop | Continuous stop logic; PWM=0; STBY LOW on fault | Watchdog timer pattern | stop=timeout OR front/side violation OR reverse/rear violation OR E-stop |
| S33 | Timing/reaction | Measured combined latency + actual speed + measured detection range | Benchmark A+B+C together; reaction distance; compare with detection/maneuver requirement | End-to-end timing + kinematics | reaction_distance=speed*total_loop_latency |

Member C acceptance: A/B pairing rejects stale data; costmap/world transform are checked; curve feasibility uses measured R_min; Pure Pursuit matches a manual case; rate limiting works; watchdog fires independently; combined loop latency and reaction distance are measured.

8. Interface Contracts and Data Ownership

| Packet | Required fields from source | Consumer |
| --- | --- | --- |
| A -> C | timestamp; traversability grid; grid resolution/scale metadata; confidence; LOW_CONFIDENCE | Stage 21 |
| B -> C | timestamp; pose x/y/z/roll/pitch/yaw; confidence; valid; optional trajectory; loop flag | Stage 21 |
| C -> Rover | rate-limited steering angle; adjusted speed; freshness needed by watchdog | ESP32 |
| Rover -> Laptop telemetry | Implementation-defined diagnostics; expose sensor distances, watchdog state, command freshness, actuator state | Logging/validation |

Member A packet (conceptual JSON-style): {timestamp, traversability_grid, grid_resolution, grid_scale, confidence, low_confidence} Member B packet: {timestamp, pose{x,y,z,roll,pitch,yaw}, pose_confidence, valid, trajectory?, loop_closure}

Ownership rule Member A owns perception only; Member B owns localization only; Member C owns costmap/planning/control; ESP32 owns final actuator gating and safety stop. This prevents responsibility overlap and makes failures diagnosable.

9. Software-Only Validation Before Any Motor Power

| Test | Input | Pass condition |
| --- | --- | --- |
| A1 Calibration | Checkerboard photos | K/D saved and reproducible. |
| A2 Preprocess | Recorded frames | Fixed-resolution undistorted frames with timestamps. |
| A3 Depth | Recorded frames | MiDaS depth + latency log. |
| A4 Geometry | Depth + K/D + mount data | 3D points/plane overlays plausible. |
| A5 Hazard | Known raised/depressed objects | SAFE/OBSTACLE/DROP_OFF signs match expected geometry. |
| A6 A packet | Recorded A packet | C can consume without A running. |
| B1 ORB-SLAM3 | Benchmark data | Build + valid tracking. |
| B2 Project camera/IMU | Captured data | Pose/tracking state + timing logs. |
| C1 Transform | Synthetic known cells | World obstacle position matches hand calculation. |
| C2 Costmap | Synthetic occupancy | Rolling window and inflation behave exactly. |
| C3 Planner | Synthetic gaps | Feasible/infeasible decisions match geometry. |
| C4 Controller | Known curve | Pure Pursuit matches manual case; rate limit blocks step. |
| C5 Watchdog | Simulated failures | Stop conditions fire deterministically. |

Build a replay harness so every code change can be run against the same captured sequences.

Log every stage boundary and save visualizations for A stages 3-10.

Unit-test formulas independently from live I/O.

Use one central configuration for K/D, mounting pose, thresholds, vehicle dimensions, steering limits, timing, and safety settings.

10. Hardware Build Inventory

| Subsystem | Final components | Critical verification |
| --- | --- | --- |
| Controller/camera | ESP32-WROOM-32; ESP32-CAM + MB; USB data cable | Verify delivered ESP32-CAM sensor label. |
| Sensors | MPU-6050; HC-SR04 x4; 1k ohm x10; 2k ohm x10 | Four Echo dividers required. |
| Drive | TT/BO motor pair 6V ~200RPM; wheels x4; TB6612FNG | Exact motor voltage/stall current must be confirmed. |
| Steering | MG90S 180-degree; horn; steering arm; tie rods; knuckles; axles/bearings | Confirm linkage mechanically fits and actual wheel angle is measured. |
| Power | 2S Li-ion 7.4V/8.4V; XL4015; LM2596; main fuse/holder; NC E-stop; rocker; PDB; 5V 2-3A protection | Fuse and buck current depend on actual motor/load verification. |
| Filtering/build | 470uF x2; 100uF x2; 0.1uF x8; perfboard; 18-20AWG; 22-24AWG; jumpers; extension; M3; acrylic; heat-shrink; ties | No Dupont for power. |
| Charging | 2S 8.4V CC/CV charger | Separate charging path; never use a 1-cell 4.2V charger. |

| Mechanical parameter | Final starting value |
| --- | --- |
| Overall length | 300 mm |
| Overall width | 200 mm |
| Wheelbase | 220 mm |
| Track width | 180 mm |
| Wheel diameter | 65-70 mm |
| Ground clearance | 30-40 mm |
| Max steering | ~25-30 degrees, measure actual wheel angle |
| Min turning radius | ~400-470 mm starting expectation; compute measured R_min |

11. Power Architecture and Safe Bring-Up

2S battery 7.4V nominal / 8.4V full -> 5A fuse (provisional until motor data) -> NC E-stop -> rocker switch -> +BUS ¦--> XL4015 -> exactly 6.00V -> TB6612 VM -> two rear motors ¦--> LM2596 -> exactly 5.00V -> WROOM, CAM, MG90S, HC-SR04 x4 Battery - -> COMMON GROUND -> both bucks, TB6612, WROOM, CAM, servo, sensors, capacitors

Build only 5V and 6V rails. Set XL4015 to exactly 6.00V and LM2596 to exactly 5.00V with a multimeter before connecting loads.

Confirm common-ground continuity.

Install 470uF across motor supply near TB6612 VM/GND; install 470uF + 100uF across 5V distribution; install 0.1uF directly across each motor.

Connect TB6612 VCC to ESP32 3.3V, VM to 6V, GND to common ground.

Use 18-20AWG for battery/buck/driver/motor paths and 22-24AWG for signals.

Fuse must be finalized only after actual motor stall current, battery discharge rating, buck capability, connector rating and wire rating are confirmed.

Power gate No downstream module gets power until the rail it sees has been measured. Pressed E-stop must remove power to both buck converters and therefore motors, boards, servo and sensors.

12. Final GPIO and Peripheral Wiring Checklist

| GPIO | Device | Function |
| --- | --- | --- |
| 4 | TB6612 BIN2 | Right direction 2 |
| 5 | Front TRIG | Front trigger |
| 13 | TB6612 PWMA | Left PWM |
| 14 | TB6612 PWMB | Right PWM |
| 16 | Front ECHO via divider | Front distance |
| 17 | Left TRIG | Left trigger |
| 18 | MG90S | Steering PWM |
| 19 | Right TRIG | Right trigger |
| 21 | MPU SDA | I2C data |
| 22 | MPU SCL | I2C clock |
| 23 | TB6612 STBY | Motor enable |
| 25 | TB6612 BIN1 | Right direction 1 |
| 26 | TB6612 AIN1 | Left direction 1 |
| 27 | TB6612 AIN2 | Left direction 2 |
| 32 | Left ECHO via divider | Left distance |
| 33 | Rear TRIG | Rear trigger |
| 34 | Right ECHO via divider | Right distance; input-only |
| 35 | Rear ECHO via divider | Rear distance; input-only |

Each HC-SR04 Echo divider: ECHO(5V) --[1k ohm]--+--> ESP32 GPIO ¦ [2k ohm] ¦ GND

Peripheral power: MG90S red=5V, brown/black=GND, signal=GPIO18. MPU-6050 VCC=3.3V, GND common, SDA=21, SCL=22, AD0=GND, INT initially unused. ESP32-CAM=5V + common GND; Wi-Fi only to laptop.

13. ESP32-WROOM Firmware State Machine

BOOT -> STBY LOW, PWM 0 -> init GPIO/I2C/ultrasonic/IMU -> connect Wi-Fi -> wait for fresh valid command VALID + SAFE -> STBY HIGH -> apply steering + motor command TIMEOUT / INVALID / UNSAFE ULTRASONIC / INTERNAL FAULT -> PWM 0 -> STBY LOW PHYSICAL E-STOP -> power removed upstream of both bucks

| Firmware module | Must implement | Test |
| --- | --- | --- |
| GPIO safety | Safe outputs at boot | STBY LOW + PWM 0 on boot. |
| IMU | I2C readout | Scanner + stable samples. |
| Ultrasonic | Sequential trigger/echo | One sensor then all four; cross-talk check. |
| Servo | GPIO18 PWM | Center + limited sweep. |
| Motor | PWMA/PWMB + direction + STBY | One motor then two wheels lifted. |
| Command | Receive + validate + timestamp | Good/bad/old packets. |
| Watchdog | Timeout + distances + E-stop | Every stop condition. |
| Telemetry | State/sensor debug data | Compare event log to physical tests. |

Use ESP32 LEDC PWM for the TB6612 PWM outputs. Keep the firmware independent from laptop planner semantics: it applies only a valid, fresh command, while the watchdog can override it.

14. ESP32-CAM Bring-Up and Camera Data Discipline

Program ESP32-CAM through the ESP32-CAM-MB.

Power it from the 5V electronics rail and common ground only; no direct WROOM GPIO link is required.

Bench-test video streaming before rover integration.

Confirm measured frame interval and log drops.

Capture a short replay dataset and drive the full software pipeline from the recording before enabling live autonomy.

Use the exact calibrated resolution for live inference and ORB-SLAM3.

Keep the camera rigidly mounted; any physical camera change invalidates K/D and mount extrinsics.

Logical path separation ESP32-CAM -> laptop video and laptop <-> ESP32-WROOM command/telemetry are independent logical paths. A camera stream remaining alive must never prevent a command timeout from stopping the rover.

15. Complete Bench-Test Sequence Before Chassis Integration

Build 5V/6V rails and verify with multimeter.

Confirm common ground continuity.

Program ESP32-WROOM; verify serial boot.

Run I2C scan and test MPU-6050.

Test one HC-SR04 through its divider; then all four sequentially.

Test MG90S center and limited steering sweep with wheels lifted.

Test TB6612 STBY: GPIO23 LOW must disable outputs.

Test one motor at low PWM, then both motors with wheels lifted.

Test Wi-Fi command reception and timeout.

Test ultrasonic and Wi-Fi safety stops at low speed.

Test ESP32-CAM streaming independently.

Only now assemble the complete vehicle and move outdoors at low speed.

16. Physical Calibration and Coordinate-Frame Lock

| Parameter | How to get it | Used by |
| --- | --- | --- |
| K,D | 15-25 checkerboard images + cv2.calibrateCamera | Undistortion, projection, essential geometry, ORB-SLAM3. |
| Camera height/tilt | Tape measure + angle tool | Scale anchoring. |
| Camera mount R,t | Measured camera pose relative chassis | Stage 7. |
| IMU mount R,t | Measured IMU pose relative chassis | Stage 12. |
| Wheelbase L | Physical measurement | Bicycle model, Pure Pursuit. |
| delta_max | Measure actual front-wheel steering angle | R_min. |
| Vehicle width | Physical body width | Footprint inflation. |
| Detection range | Measured on real camera/terrain | Stage 33. |
| T_up/T_down | Flat-ground noise experiment | Stage 9. |
| delta_t_sync | Synchronization test | Stage 21. |

Lock these values into a versioned calibration/configuration file. Never leave critical geometry in source code literals.

17. Hardware-in-the-Loop Integration

Camera stream live, rover stationary.

Command path live, motors disabled; verify packet freshness and ESP32 acceptance.

Run A+B+C from recorded input and verify real command emission.

Lift wheels; verify motor directions/PWM and steering sign.

Ground test at very low speed with a simple path.

Run a far-obstacle avoidance test and inspect costmap/curve/steering.

Intentionally break Wi-Fi; rover must stop locally.

Intentionally place a close obstacle in front/side; watchdog must stop.

Perform one reverse-and-retry with rear clearance.

Perform blocked-rear recovery; rover must refuse reverse and stop.

Run measured Point-A-to-Point-B trials and save all logs.

18. Test Course, Ground Truth, and Metrics

Design one controlled course and reuse it for localization, planning and navigation. Measure the course physically before any metrics are reported.

| Metric | Formula | Evidence |
| --- | --- | --- |
| ATE RMSE | sqrt((1/N) * sum ¦¦p_est_i-p_gt_i¦¦^2) | Tape-measured checkpoints and timestamp alignment. |
| RPE translation | sqrt((1/M) * sum ¦¦trans(E_i)¦¦^2) | Short-term drift between nearby poses. |
| SPL | (1/N) * sum [S_i * (L_i/max(P_i,L_i))] | Measured shortest path L_i and actual driven path P_i. |
| Reaction distance | vehicle_speed * total_loop_latency | Measured combined A+B+C latency at actual test speed. |

Make start=(0,0,0) and forward direction +x.

Measure checkpoint positions and times.

Include a revisit segment to demonstrate loop closure.

Measure obstacle positions, corridor widths and shortest feasible start-goal path L_i.

Use the same test-course geometry for SPL and repeated runs.

Log frame ID, timestamp, pose, confidence, feature/match count, loop event, ground truth, commands, watchdog state and success/failure.

19. Outdoor Dataset Validation - RUGD and RELLIS-3D

Select representative outdoor scenes with ground, rocks, vegetation, uneven terrain, shadows and difficult lighting.

Run the unchanged Member A Stages 3-10 configuration.

Do not tune per-image thresholds.

Save original image, MiDaS depth, RANSAC inlier visualization, signed-distance overlay, grid and confidence.

Compare predicted regions with available labels, or documented qualitative review where labels do not exist.

Record failure patterns such as grass vs obstacle, shadow vs drop-off, low-texture confidence loss, or missed thin vegetation.

Archive 3-5 representative successes and failures for the final report/pitch.

Claim boundary Dataset validation proves the pipeline was exercised outside the team course. It does not prove universal outdoor performance or safety.

20. Failure Modes -> Detection -> Response

| Failure | Detection | Response |
| --- | --- | --- |
| A/B timestamps mismatch | No B pose within tolerance | Discard pair; no costmap update. |
| Low A/B confidence | LOW_CONFIDENCE / low pose confidence | Reduce trust; slow/hold safe state per policy. |
| Narrow corridor | Inflation leaves no feasible gap | Reverse once; then stop if still blocked. |
| Curve too tight | Curvature violates R_min | Reject curve; replan/recover. |
| Close obstacle | Distance inside reaction region | Independent watchdog stop. |
| Wi-Fi loss | No fresh command by timeout | PWM 0; STBY LOW. |
| Jerky steering | Step change > max_rate | Rate limiter caps delta. |
| Ultrasonic spike | Single inconsistent reading | Debounce across consecutive readings. |
| Blocked rear | Rear sensor/costmap occupied | Refuse reverse; stop. |
| No clear local gap | No feasible side | Slow/stop. |
| Compute lag | Cycle > live rate | Watchdog can stop; benchmark combined loop. |
| Camera moved | Geometry inconsistent | Stop and recalibrate. |
| IMU/camera desync | Pose jumps/inconsistent fusion | Return to Stage 12 validation. |

21. SIH Demonstration and Evidence Package

| Demo | Show | Proves |
| --- | --- | --- |
| Perception | Camera + MiDaS + hazard/traversability overlay | Path/hazard perception. |
| Localization | Pose trajectory + confidence + revisit/loop closure | GPS-denied visual-inertial localization. |
| Avoidance | Obstacle -> costmap -> gap -> cubic curve -> steering | Dynamic routing around obstacle. |
| Actuation | Steering/motor motion follows command | Planner-to-rover interface. |
| Safety | Kill Wi-Fi / close obstacle / press E-stop | Independent stop authority. |
| Point A -> B | Repeated measured run | End-to-end PS success criterion. |
| Validation evidence | RUGD/RELLIS + ATE/RPE + SPL + latency | Quantitative support for claims. |

Recommended claim Say "hybrid AI + classical robotics + deterministic safety." Do not say every stage is AI. MiDaS-Small is the principal neural perception model; ORB/geometry/planning/control/safety are distinct algorithmic layers.

22. Consolidated Formula Reference

| Topic | Formula |
| --- | --- |
| K | [[fx,0,cx],[0,fy,cy],[0,0,1]] |
| D | [k1,k2,p1,p2,k3] |
| Texture | variance(¦gradient(I)¦) |
| Scale | s=median(depth_expected/depth_midas_raw); d=s*d_raw |
| Pinhole | X=(u-cx)Z/fx; Y=(v-cy)Z/fy; Z=depth(u,v) |
| Rigid 3D | P_vehicle=R*P_camera+t |
| Plane | aX+bY+cZ+d=0 |
| Signed distance | dist=(aX+bY+cZ+d)/sqrt(a^2+b^2+c^2) |
| Extrinsic | T_imu_camera=inv(T_chassis_imu)*T_chassis_camera |
| Epipolar | p2^T E p1=0; E=K^T F K; E=[t]x R |
| BA | min sum ¦¦project(point_j,pose_i)-observed_pixel_ij¦¦^2 |
| World transform | xw=xt+xl cos(theta)-yl sin(theta); yw=yt+xl sin(theta)+yl cos(theta) |
| Rolling window | keep if distance(cell,vehicle)<=R_local; R_local>=detection+reaction+margin |
| Log odds | l_t=l_(t-1)+log(p/(1-p))-l_0 |
| Inflation | r_inflate=vehicle_width/2+safety_margin |
| Turning radius | R_min=L/tan(delta_max) |
| Cubic | y(x)=a x^3+b x^2+c x+d |
| Pure Pursuit | delta=atan(2L sin(alpha)/l_d) |
| Rate limiting | delta_app=delta_prev+clip(delta_des-delta_prev,-max_rate*dt,+max_rate*dt) |
| Watchdog | stop=timeout OR front/side violation OR reverse/rear violation OR E-stop |
| Reaction | reaction_distance=vehicle_speed*total_loop_latency |
| ATE | sqrt((1/N)*sum¦¦p_est-p_gt¦¦^2) |
| RPE | E_i=inv(T_gt_i)T_gt_(i+delta) inv(inv(T_est_i)T_est_(i+delta)); RPE_trans=sqrt((1/M)sum¦¦trans(E_i)¦¦^2) |
| SPL | (1/N)*sum[S_i*(L_i/max(P_i,L_i))] |

23. Master Definition-of-Done Checklist

Software

[ ] Environment reproducible

[ ] Camera replay works

[ ] Logging works

[ ] Config centralized

Member A

[ ] K/D saved

[ ] MiDaS real frames

[ ] Scale measured

[ ] RANSAC visualized

[ ] T_up/T_down measured

[ ] A packet replay

Member B

[ ] ORB-SLAM3 builds

[ ] Benchmark run

[ ] Extrinsics measured

[ ] Time sync validated

[ ] Live project tracking

[ ] Confidence/invalid state

[ ] Loop closure

Member C

[ ] Timestamp gate

[ ] World transform

[ ] Rolling window

[ ] Footprint inflation

[ ] R_min measured

[ ] Cubic curve verified

[ ] Pure Pursuit manual check

[ ] Rate limiter

[ ] Recovery one retry

Hardware

[ ] 6V/5V rails measured

[ ] Common ground

[ ] 4 Echo dividers

[ ] IMU

[ ] 4 ultrasonics

[ ] Servo

[ ] STBY safety

[ ] Motors

[ ] Camera stream

Integration

[ ] Wi-Fi timeout

[ ] Front/side stop

[ ] Rear reverse stop

[ ] Invalid command stop

[ ] E-stop

[ ] Combined latency

[ ] Reaction distance

[ ] Detection range requirement

SIH

[ ] RUGD/RELLIS evidence

[ ] ATE/RPE

[ ] SPL with measured L_i

[ ] Repeated A->B runs

[ ] 3-5 success/failure examples

24. The Exact Day-to-Day Implementation Sequence

| Order | Concrete action | Deliverable |
| --- | --- | --- |
| 1 | Create repo/config/logging and install toolchains | Reproducible laptop environment. |
| 2 | Implement input abstraction and timestamped replay | Camera/IMU replay harness. |
| 3 | Run checkerboard calibration | K/D + calibration metadata. |
| 4 | Implement Stage 1-11 Member A | Depth + grid + packet + visual artifacts. |
| 5 | Build ORB-SLAM3 and run benchmark | Known-good SLAM binary and logs. |
| 6 | Implement Stage 12-20 Member B adapter | Synchronized pose packet + confidence. |
| 7 | Implement Stage 21-30 Member C with synthetic packets | Costmap/planner/controller tests. |
| 8 | Implement Stage 31 transport abstraction and command freshness | Laptop-to-rover command interface. |
| 9 | Implement ESP32 firmware and watchdog | Safe low-level controller. |
| 10 | Build rails and verify power | Measured 5V/6V, continuity. |
| 11 | Bring up sensors/servo/motors individually | Bench validation logs. |
| 12 | Bring up ESP32-CAM stream | Camera stream evidence. |
| 13 | Calibrate mounted camera/IMU | Physical extrinsics + geometry. |
| 14 | Hardware-in-the-loop stationary | Command/safety integration. |
| 15 | Low-speed outdoor integration | First complete closed loop. |
| 16 | Measure full latency/reaction distance | Stage 33 evidence. |
| 17 | Run controlled course repeatedly | ATE/RPE/SPL/success data. |
| 18 | Run RUGD/RELLIS validation | Generalization evidence. |
| 19 | Rehearse SIH demo and failure demo | Repeatable pitch evidence. |

Do not bypass steps The point of software-first development is that when the physical rover misbehaves, you already know whether the source is perception, localization, planning, command transport, firmware, wiring, or power. That separation is lost if everything is built at once.

25. Source References and Final Schematic

| Reference | Use |
| --- | --- |
| S1 - SIH PS 26126 | Problem statement requirements and success criterion. |
| S2 - FINAL_LOCKED_UGV_BUILD_DOCUMENT(2) | Final power/GPIO/mechanics/safety/procurement checks. |
| S3 - software_validation_rover_final_design | Firmware state machine, bench sequence, physical placement. |
| S4 - Wire-to-Wire_Pipeline_A-B-C_FINAL(4) | All stages, algorithms, math, interfaces, validation/failure modes. |
| S5 - FINAL_UGV_BOM_PRICES_ALL_CONNECTIONS_ARCHITECTURE(1) | BOM and full connection cross-check. |
| External toolchain sources | Official MiDaS, ORB-SLAM3, and Espressif documentation used only to acquire/build software; exact checkpoint/version must be frozen locally. |

Supplied final schematic (visual reference):

26. Final Sign-Off and Build Record

Status ARCHITECTURE: FROZEN SOFTWARE PIPELINE: FULLY SPECIFIED HARDWARE WIRING: FROZEN IMPLEMENTATION: TO BE EXECUTED PURCHASE-TIME CHECKS: REQUIRED RUNTIME PERFORMANCE: MUST BE MEASURED SIH FIELD SUCCESS: MUST BE DEMONSTRATED

This roadmap is intentionally explicit about uncertainty. A module is not marked "done" because it is described; it is done only after the specified test produces evidence. That is the standard to use throughout implementation and in the final SIH presentation.

| Sign-off | Owner | Date | Evidence |
| --- | --- | --- | --- |
| Environment |  |  |  |
| Member A |  |  |  |
| Member B |  |  |  |
| Member C |  |  |  |
| ESP32 firmware |  |  |  |
| Hardware/power |  |  |  |
| HIL + safety |  |  |  |
| Outdoor validation |  |  |  |
| Final SIH demo |  |  |  |

End of roadmap.