Software-Validation Ackermann Rover

Design status

This document freezes the software-validation platform architecture and GPIO map. It is a low-speed 2WD rear-drive, front-steered Ackermann rover designed to validate the laptop perception, localization, planning, Pure Pursuit, command, watchdog, and safety interfaces described in the project pipeline.

Important electrical condition: The target motor is a 6 V, approximately 200 RPM TT/BO geared DC motor, but the exact purchased motor must have its voltage and stall-current specification checked before buying the final fuse and motor buck converter. Do not buy a motor merely because a listing title says “200 RPM.” The TB6612FNG must never be subjected to a prolonged stall.

System architecture

+------------------------------------+
                         |              LAPTOP                |
                         |                                    |
                         | Camera receiver                    |
                         | MiDaS-Small perception             |
                         | ORB-SLAM3 / localization           |
                         | Costmap + local planning           |
                         | Pure Pursuit                       |
                         | Steering + speed command generator |
                         +----------------+-------------------+
                                          | Wi-Fi
             Camera stream               | command packets
       +--------------------+             |
       |  ESP32-CAM OV2640  |-------------+
       |  camera stream     |
       +--------------------+
                  | 5 V / GND
                  |
+--------------------------------------------------------------------------+
|                       ESP32-WROOM-32 ROVER                              |
|  Receives laptop commands; reads IMU and ultrasonics; enforces safety;  |
|  controls TB6612FNG drive and MG90S front steering.                     |
+----------------------+---------------------+----------------------------+
                       |                     |
                 steering PWM           motor direction/PWM + STBY
                       |                     |
                 +-----v-----+       +------v--------+
                 |   MG90S   |       |   TB6612FNG   |
                 | front     |       | dual driver   |
                 | steering  |       +---+--------+--+
                 +-----------+           |        |
                                   left rear     right rear
                                   TT/BO motor  TT/BO motor

        ESP32-WROOM also connects to:
  - MPU-6050 via I2C
  - HC-SR04 front, left, right, and rear sensors

The laptop runs the compute-heavy pipeline. The ESP32-CAM is an independent Wi-Fi camera streamer. The ESP32-WROOM is the low-level vehicle controller and is the final authority for motor enable/disable.

Vehicle configuration

- Kinematics: 2WD rear-wheel drive, front-wheel Ackermann steering.

- Rear drive: two DC geared motors, one at each rear wheel.

- Steering: one MG90S servo operating both front wheels through tie rods and steering knuckles.

- Recommended first chassis material: 3–5 mm acrylic, with M3 hardware.

- Suggested starting geometry: 300 mm overall length, 200 mm overall width, 220 mm wheelbase, 180 mm track width.

- Target speed: low speed during first tests; tune speed only after the steering, safety logic, and vehicle stability are proven.

Do not use a four-fixed-wheel 4WD skid-steer chassis for this software stack: the project control pipeline outputs steering angle and assumes the bicycle/Ackermann model.

Final component list

Controllers and camera

| # | Component | Quantity | Original SKU / sourcing note |
| --- | --- | --- | --- |
| 1 | ESP32-WROOM-32 DevKit, 30/38-pin development board | 1 | Robu SKU 835800 listed by user |
| 2 | ESP32-CAM board with OV2640 or confirmed OV3660 sensor | 1 | Robu SKU 456154; confirm actual camera sensor on the delivered listing/label |
| 3 | ESP32-CAM-MB USB programmer | 1 | Robu SKU 1150750 |
| 4 | USB data cable for ESP32-WROOM programming | 1 | Ensure it is a data cable, not charge-only |

Sensors

| # | Component | Quantity | Note |
| --- | --- | --- | --- |
| 5 | MPU-6050 accelerometer + gyroscope breakout, 3.3 V-compatible | 1 | I2C IMU |
| 6 | HC-SR04 ultrasonic range sensor | 4 | Front, left, right, rear |
| 7 | 1 kΩ, 1/4 W resistor | 10 | Four required for Echo dividers; keep spares |
| 8 | 2 kΩ, 1/4 W resistor | 10 | Four required for Echo dividers; keep spares |

Drive and steering

| # | Component | Quantity | Requirement |
| --- | --- | --- | --- |
| 9 | TT/BO geared DC motor | 2 | Target: 6 V nominal, approximately 200 RPM, matched pair; verify exact stall current and motor voltage before purchase |
| 10 | TT/BO compatible 65–70 mm wheels | 4 | Two rear powered wheels and two front free/steerable wheels |
| 11 | TB6612FNG dual DC motor-driver breakout | 1 | Use only for a lightweight, low-speed rover; nominally about 1.2 A continuous per channel, higher peak current only briefly |
| 12 | MG90S 180-degree servo | 1 | Steering servo; verify supplied horn fits the linkage |
| 13 | Servo horn, steering arm, tie rods, steering knuckles, front axles/bearings | 1 set | Required to construct the Ackermann front end |

Power and safety

| # | Component | Quantity | Requirement |
| --- | --- | --- | --- |
| 14 | 7.4 V nominal 2S Li-ion battery pack with BMS/protection | 1 | User-selected Robu SKU 797324; check discharge rating and connector |
| 15 | 6.0 V DC-DC buck converter for the motor rail | 1 | Output-current target at least 3 A; choose 5 A rated only after confirming motor current; adjustable and tested using a multimeter |
| 16 | 5.0 V DC-DC buck converter for electronics | 1 | Genuine 3 A or higher continuous capability strongly preferred |
| 17 | Main inline DC fuse holder + fuse | 1 | Provisional 5 A slow-blow candidate only; finalize after confirming motor current, buck capability, battery rating, and wire rating |
| 18 | Normally-closed emergency-stop switch, DC-rated | 1 | In series with battery positive, before both buck converters |
| 19 | DC-rated rocker master switch | 1 | User-selected Robu SKU R132455 only if its current rating suits the final system |
| 20 | Positive and ground distribution block / terminal block | 1 | Creates clean power and common-ground buses |
| 21 | 5 V rail protection (2–3 A resettable polyfuse or small fuse) | 1 | Recommended for electronics branch |

Filtering and construction

| # | Component | Quantity | Requirement |
| --- | --- | --- | --- |
| 22 | 470 µF electrolytic capacitor, 16 V minimum | 2 | One near 5 V distribution; one across a buck input/motor rail as needed |
| 23 | 100 µF electrolytic capacitor, 10–16 V | 2 | 5 V rail filtering |
| 24 | 0.1 µF ceramic capacitor, 50 V | 8 | Two needed across motor terminals; remaining parts for local decoupling/spares |
| 25 | Perfboard, around 5 × 7 cm | 1 | Echo voltage dividers and power filtering |
| 26 | 18–20 AWG stranded wire | About 3 m | Battery, fuse, E-stop, switch, buck input/output, driver-to-motor wiring |
| 27 | 22–24 AWG stranded wire | About 3 m | Signals and low-current connections |
| 28 | Dupont jumper assortment (M-M, M-F, F-F) | 1 set | Signals only; never use for battery/motor power |
| 29 | Servo extension cable | 1 | If servo lead cannot reach control board |
| 30 | M3 screws, nuts, washers, and standoffs | 1 set | Chassis and board mounting |
| 31 | 3–5 mm acrylic sheet | 1 set | Main base/top plate and brackets |
| 32 | Heat-shrink tubing | 1 set | Insulate all soldered power joints |
| 33 | Zip ties | 1 pack | Cable strain relief and routing |
| 34 | Checkerboard print, 25 mm squares | 1 | Camera calibration |

Purchases that remain conditional

Do not regard these values as final until the exact motor listing/datasheet is confirmed:

- Exact TT/BO motor model and its stall current.

- 6 V motor-buck output-current rating.

- Main fuse type and rating.

- Battery discharge capability and connector current rating.

Power architecture

2S BATTERY (+), 7.4 V nominal / 8.4 V fully charged
        |
        +--> MAIN DC FUSE (provisional until motor data is confirmed)
                |
                +--> NORMALLY-CLOSED E-STOP
                        |
                        +--> ROCKER MASTER SWITCH
                                |
                                +--> POSITIVE POWER BUS
                                      |                         |
                                      |                         |
                                      v                         v
                              6.0 V MOTOR BUCK          5.0 V ELECTRONICS BUCK
                              input from bus             input from bus
                                      |                         |
                                      v                         +--> 5 V BUS
                              TB6612FNG VM                    |      |      |      |
                                      |                         |      |      |      |
                                      v                         |      |      |      +--> HC-SR04 VCC x4
                              Left and right motors             |      |      +---------> MG90S red wire
                                                                |      +--------------> ESP32-CAM 5 V
                                                                +---------------------> ESP32-WROOM 5 V/VIN

        2S BATTERY (-) --> COMMON GROUND BUS
                         |        |       |       |        |       |       |
                         |        |       |       |        |       |       +--> MPU-6050 GND
                         |        |       |       |        |       +----------> HC-SR04 GND x4
                         |        |       |       |        +------------------> MG90S brown wire
                         |        |       |       +---------------------------> ESP32-CAM GND
                         |        |       +-----------------------------------> ESP32-WROOM GND
                         |        +-------------------------------------------> TB6612FNG GND
                         +-----------------------------------> 5 V and 6 V buck GND terminals

Voltage domains

| Rail | Supplies |
| --- | --- |
| Battery input: 7.4 V nominal, 8.4 V maximum after charging | Inputs of both buck converters |
| 6.0 V motor rail | TB6612FNG VM only, then the two drive motors |
| 5.0 V electronics rail | ESP32-WROOM 5 V/VIN, ESP32-CAM 5 V, MG90S, HC-SR04 VCC x4 |
| ESP32 3.3 V regulator output | TB6612FNG VCC and MPU-6050 VCC |

Never feed 5 V into an ESP32 3.3 V pin. Never feed the 6 V motor rail into TB6612FNG VCC; VM is motor supply and VCC is 3.3 V logic supply.

Frozen ESP32 GPIO map

| ESP32-WROOM pin | Connected item | Function |
| --- | --- | --- |
| GPIO 4 | TB6612FNG BIN2 | Right motor direction 2 |
| GPIO 5 | Front HC-SR04 TRIG | Front trigger output |
| GPIO 13 | TB6612FNG PWMA | Left motor PWM |
| GPIO 14 | TB6612FNG PWMB | Right motor PWM |
| GPIO 16 | Front HC-SR04 ECHO through divider | Front distance input |
| GPIO 17 | Left HC-SR04 TRIG | Left trigger output |
| GPIO 18 | MG90S signal | Steering PWM |
| GPIO 19 | Right HC-SR04 TRIG | Right trigger output |
| GPIO 21 | MPU-6050 SDA | I2C data |
| GPIO 22 | MPU-6050 SCL | I2C clock |
| GPIO 23 | TB6612FNG STBY | Formal motor-enable signal |
| GPIO 25 | TB6612FNG BIN1 | Right motor direction 1 |
| GPIO 26 | TB6612FNG AIN1 | Left motor direction 1 |
| GPIO 27 | TB6612FNG AIN2 | Left motor direction 2 |
| GPIO 32 | Left HC-SR04 ECHO through divider | Left distance input |
| GPIO 33 | Rear HC-SR04 TRIG | Rear trigger output |
| GPIO 34 | Right HC-SR04 ECHO through divider | Right distance input; input-only pin |
| GPIO 35 | Rear HC-SR04 ECHO through divider | Rear distance input; input-only pin |
| 5 V/VIN | 5 V electronics bus | Board power input, according to board label |
| 3.3 V | TB6612FNG VCC and MPU-6050 VCC | Low-current logic/sensor supply |
| GND | Common ground bus | Ground reference |

GPIO 34 and GPIO 35 are input-only, which is appropriate for ultrasonic Echo signals. Do not use thin Dupont leads for battery, buck, motor-driver, or motor wiring.

Complete pin-to-pin wiring

TB6612FNG motor driver

| TB6612FNG pin | Connect to | Details |
| --- | --- | --- |
| VM | 6.0 V motor-buck VOUT+ | 18–20 AWG power wire |
| VCC | ESP32 3.3 V | Logic supply only |
| GND | Common ground bus | Must share ground with ESP32 and both bucks |
| STBY | ESP32 GPIO 23 | Motor-enable output: LOW disables motor outputs, HIGH enables |
| PWMA | ESP32 GPIO 13 | LEDC PWM for left motor |
| AIN1 | ESP32 GPIO 26 | Left direction input 1 |
| AIN2 | ESP32 GPIO 27 | Left direction input 2 |
| AO1 / A01 | Left rear motor terminal 1 | Swap AO1/AO2 if forward direction is reversed |
| AO2 / A02 | Left rear motor terminal 2 |  |
| PWMB | ESP32 GPIO 14 | LEDC PWM for right motor |
| BIN1 | ESP32 GPIO 25 | Right direction input 1 |
| BIN2 | ESP32 GPIO 4 | Right direction input 2 |
| BO1 / B01 | Right rear motor terminal 1 | Swap BO1/BO2 if forward direction is reversed |
| BO2 / B02 | Right rear motor terminal 2 |  |

Formal STBY safety behavior

Power-on / boot:     STBY = LOW
Initialize sensors:  STBY = LOW
Connect Wi-Fi:       STBY = LOW
Valid command + safety healthy: STBY = HIGH
Wi-Fi timeout:       stop PWM, STBY = LOW
Unsafe ultrasonic reading: stop PWM, STBY = LOW
Invalid command or software fault: stop PWM, STBY = LOW

The physical E-stop remains independent: it cuts power before both buck converters.

MG90S steering servo

| MG90S wire | Connect to |
| --- | --- |
| Brown / black | Common ground bus |
| Red | 5.0 V electronics bus |
| Orange / yellow | ESP32 GPIO 18 |

The servo must receive power from the 5 V buck, not from an ESP32 3.3 V pin or GPIO pin.

MPU-6050

| MPU-6050 pin | Connect to |
| --- | --- |
| VCC | ESP32 3.3 V pin |
| GND | Common ground bus |
| SDA | ESP32 GPIO 21 |
| SCL | ESP32 GPIO 22 |
| AD0 | GND unless a second MPU-6050 is installed |
| INT | Leave unconnected initially; optional future interrupt input |

Front HC-SR04

| HC-SR04 pin | Connect to |
| --- | --- |
| VCC | 5.0 V electronics bus |
| GND | Common ground bus |
| TRIG | ESP32 GPIO 5 |
| ECHO | Echo-divider input; divider output goes to GPIO 16 |

Left HC-SR04

| HC-SR04 pin | Connect to |
| --- | --- |
| VCC | 5.0 V electronics bus |
| GND | Common ground bus |
| TRIG | ESP32 GPIO 17 |
| ECHO | Echo-divider input; divider output goes to GPIO 32 |

Right HC-SR04

| HC-SR04 pin | Connect to |
| --- | --- |
| VCC | 5.0 V electronics bus |
| GND | Common ground bus |
| TRIG | ESP32 GPIO 19 |
| ECHO | Echo-divider input; divider output goes to GPIO 34 |

Rear HC-SR04

| HC-SR04 pin | Connect to |
| --- | --- |
| VCC | 5.0 V electronics bus |
| GND | Common ground bus |
| TRIG | ESP32 GPIO 33 |
| ECHO | Echo-divider input; divider output goes to GPIO 35 |

Echo voltage-divider circuit — build four copies

The HC-SR04 Echo output is 5 V. ESP32 GPIO pins are 3.3 V logic. Every Echo line must pass through its own divider.

HC-SR04 ECHO (5 V)
       |
       +---[ 1 kΩ ]---+----> ESP32 Echo GPIO
                      |
                    [ 2 kΩ ]
                      |
                     GND

Build one divider each for GPIO 16, GPIO 32, GPIO 34, and GPIO 35. The divider output is approximately 3.3 V for a 5 V Echo signal.

ESP32-CAM

| ESP32-CAM pin | Connect to |
| --- | --- |
| 5 V | 5.0 V electronics bus |
| GND | Common ground bus |

Program the ESP32-CAM through the ESP32-CAM-MB. No ESP32-WROOM-to-ESP32-CAM GPIO connection is required for the selected architecture; both communicate with the laptop through Wi-Fi.

Capacitor and noise suppression layout

At the 5 V power distribution point

5 V BUS ----+---- (+) 470 µF electrolytic
            |
            +---- (+) 100 µF electrolytic

        GND BUS ----+---- (-) 470 µF electrolytic
            +---- (-) 100 µF electrolytic

Place these close to the servo/ESP32 power distribution point. Observe electrolytic capacitor polarity.

Near motor supply / driver

- Add one 470 µF, 16 V or higher electrolytic capacitor across the appropriate buck output or close to TB6612FNG VM and GND.

- Add one 0.1 µF ceramic capacitor directly across each brushed motor’s two terminals.

- Keep motor wires physically separated from MPU-6050 and I2C wiring where possible.

Motor terminal 1 ----+----||----+---- Motor terminal 2
                     |   0.1 µF |
                     +----------+

Physical placement

FRONT
              +-------------------------------------+
              |  Front HC-SR04                      |
              |  ESP32-CAM, rigid forward/downward  |
              |                                     |
              |     O                   O           |
              |   front steerable wheels             |
              |        tie rods + MG90S servo       |
              |                                     |
              |  Left sensor     ESP32-WROOM        |  Right sensor
              |                  MPU-6050           |
              |                                     |
              |  Battery low and near chassis center|
              |  Buck converters / power bus        |
              |                                     |
              |  TB6612FNG near rear motors         |
              |     O                   O           |
              |   left rear drive    right rear drive|
              |          Rear HC-SR04                |
              +-------------------------------------+
                                REAR

- Rigidly mount the camera; calibration becomes invalid if its pose changes.

- Mount the MPU-6050 close to the vehicle center and away from motor vibration.

- Mount the battery low and central to reduce tipping.

- Keep the motor driver close to the motors, but away from the IMU and camera wiring.

- Trigger HC-SR04 sensors sequentially in firmware; never trigger all four together because of ultrasonic cross-talk.

Bench-test sequence before chassis integration

- Build only the 5 V and 6 V rails; set each output with a multimeter before connecting electronics.

- Confirm common ground continuity.

- Program ESP32-WROOM; verify serial boot.

- Run I2C scanner, then test MPU-6050.

- Test one HC-SR04 using its Echo divider, then test all four sequentially.

- Test servo centre and limited steering sweep with wheels lifted.

- Test TB6612FNG STBY: GPIO 23 LOW must stop/disable outputs.

- Test one motor at low PWM, then both motors with wheels lifted.

- Test Wi-Fi command reception and timeout behavior.

- Test ultrasonic and Wi-Fi timeout stops at low speed.

- Independently test ESP32-CAM streaming to laptop.

- Only then assemble the full vehicle and start low-speed outdoor tests.

Firmware safety requirements

The ESP32 must maintain an explicit safety state machine. At boot it must keep GPIO 23 LOW. It may set GPIO 23 HIGH only after initialization is complete and a valid, recent command is present.

Immediately issue motor PWM = 0 and GPIO 23 = LOW when any of these occurs:

- Wi-Fi command timeout.

- Front or side ultrasonic safety violation while moving forward.

- Rear ultrasonic safety violation while reversing.

- Invalid/out-of-range command.

- Sensor/initialization failure chosen as a stop condition.

- Internal firmware fault.

- Manual E-stop is pressed (physical power is removed regardless of firmware).

Use ESP32 LEDC PWM for the TB6612FNG PWM pins rather than depending on generic Arduino analogWrite() behavior.

Final compatibility verdict

This component architecture is suitable for a low-speed software-validation rover, provided that:

- The final motor is truly rated for 6 V and its stall current is checked.

- The motor buck converter is rated for the actual two-motor load with margin.

- The TB6612FNG is not used in prolonged stall/high-load conditions.

- The 5 V buck is a genuinely capable, stable 3 A or higher unit.

- All HC-SR04 Echo outputs use the specified dividers.

- All grounds are common.

- The final fuse is selected after checking motor, battery, buck, connector, and wire specifications.

The platform is deliberately a first-stage prototype. After software validation, upgrade priorities are: an encoder-equipped motor pair, a higher-current MOSFET motor driver if necessary, a stronger steering servo if linkage force demands it, and a more rugged metal/3D-printed chassis.